#### Script

Folding stepladders are an extremely convenient way of accessing work which is out of reach but familiarity can lead to carelessness. Falling off a stepladder is no less serious than off an ordinary ladder, equal care is called for. (The floor is just as hard).

Step ladders are covered by the same regulations as ordinary ladders regarding construction and materials and this is even more critical because of the extra parts required to make them foldable are also required to make them safe.

Establishing a habit of checking off a mental list each time a pair of steps is used, will lead to safe working. Steps are probably the most “borrowed” item of all equipment, however, just because they are on available, does not mean they are safe.

#### Key Points to consider are:

- Steps must be suitable. “Domestic” weight steps are not up to “trade” use.

- Check anti-spread device (cords, clips, brackets etc,). Remember, if it is defective the steps are condemned.

- Check folding mechanism (hinges, pin, rivets, etc,).

Remember

- CHECK – if the task requires access equipment rather than a stepladder, STOP – ask your Manager or Supervisor for guidance, don’t use a stepladder unless it is safe to do so.

- Only use step ladders where there is no safer alternative such as working off a platform or floor structure designed for safe access.

- Always spread the ladder to its fullest extend, so that it can’t suddenly move out while you are on it.

- Ensure that all four stiles are on firm, level ground. If this can not be achieved then a stepladder is not the correct equipment to use.

- You must always have secure handhold above you. Clearly this means that you cannot stand on the top three steps unless the steps are designed with a platform and handhold.

- Place the stepladder so that you do not have to reach or stretch out beyond the design of the steps.

- An assistant standing on the bottom step lowers the centre of gravity very effectively, but make sure they understand and so do not step off suddenly.

Any Questions?

| Toolbox Talk - Working at Height – Stepladders |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Location |  | Date |  | Presented By |  |
| Attendees |  |  |  |  |  |
| Name |  |  |  | Signature |  |

Notes from Presentation: